Lab 12:

MOSFET Amplifier

Name: Wan-Yu Liao

ECEN 325 Section 514

TA: Mandela

Lab Date: November 22, 2019

Lab Report Due Date: November 27, 2019

Calculation

I had the wrong value for Rs.

Rs should be 0.6V/IDA = 0.6/42uA ≈ 16kΩ

Simulation

Figure 1: DC Solution for MOSFET amplifier ▲

| VRG1 = 7.63V | VRS = 0.11V | ID1 = 8.46mA |
| --- | --- | --- |
| VRG2 = 2.37V | VRD = 1.38V | ID2 = 338pA |
| VRG3 = 6.48V |  | ID3 = 373pA |
| VRG4 = 3.52V |  |  |

For this circuit, I got a gain of 0. I rework the circuit.

Figure 1.1: New circuit for MOSFET amplifier ▲

Figure 2.1: AC Simulation of AV for MOSFET amplifier ▲

AV = 2.8033

Even I rework the circuit, the best gain I can get is 2.8033.

Figure 2.2: AC Simulation of Ri for MOSFET amplifier ▲

Ri = 7.3469kΩ

Figure 3: Time-domain waveform of Vi = 40mV 5kHz for MOSFET amplifier ▲

AV ≈2

Figure 4: Total harmonic distortion (THD) for MOSFET amplifier ▲

THD = 0.5053%

New Simulation

I re-work the simulation using the values I got from lab measurement. However, I still couldn’t get it to work.

Figure 5.1: Time-domain waveform at stage 1 of Vi = 40mV 5kHz for MOSFET amplifier ▲

AV ≈0

Figure 5.2: Time-domain waveform of Vi = 40mV 5kHz for MOSFET amplifier ▲

AV ≈0

Measurement

| CD4007P | 2N7000G | RD = 157kΩ | RRG1 = 46kΩ |
| --- | --- | --- | --- |
| λ = 0.0968 |  | RS = 16kΩ | RRG2 = 14kΩ |
| β = 1.11mA/V2 | β = 95.464mA/V2 | RX = 12Ω | RRG3 = 650Ω |
| VTh = 1.5V | VTh = 2V |  | RRG4 = 350Ω |

Table 1: Values used for MOSFET amplifier ▲

Figure 6.1: AV for MOSFET amplifier ▲

AV = 10^(-1.3112/20) = 0.8599

Figure 6.2: Ri for MOSFET amplifier ▲

Ri / (11k+ Ri) = 10^(-5.911/20)

Ri = 11.267kΩ

Figure : Time-domain waveform at stage 1 of Vi = 40mV 5kHz for MOSFET amplifier ▲

AV =1.01550.03971≈25.58

Figure 4: Total harmonic distortion (THD) at Stage 1 for MOSFET amplifier ▲

THD = 92.05%

Table

|  | Calculation (without Lambda) | Calculation (without Lambda) | Simulation | Measurement |
| --- | --- | --- | --- | --- |
| Gain | 50 | 28 | 0 | 25.58 |
| Ri | 11kΩ | 11kΩ | 7.34kΩ | 11.267kΩ |

Stage 1

Comment

Lambda value (λ):

There is a lambda value (λ) in CD4007P that would affect the output gain. λ can be found using the characteristic circuit to find the early voltage (VA),  λ = 1VA. This λ value would affect ro and ro would affect the gain. (ro=1λ*ID1 ; ro||RDRD*ideal gain=real gain)

In my case, I got λ = 0.0968 and my RD = 157kΩ and ID1 = 42uA. I used 50 as my ideal gain and calculated my real gain would be 28.96. In the measurement, I got a gain of 25.58 which is close to 28.96. In order for me to get a gain of 50 in measurement, I will need to use 89 as my ideal gain to calculate all the resistor values I need to use.

Measurement:

After I measured the stage 1 with a gain of 25, I connected it to stage 2. However, it didn’t not work as expected to give me a gain of 25. It was much lower that 25. This might have the same reason as lab 10 where Analog Discovery 2 can only handle up to 250mW. To make circuit have a gain of 25, the circuit needs to be modified to reduce the power consumption at stage 1.